Codes and data in this folder aim to demonstrate

1.how to detect EMG artifacts in scalp recordings and

2. how to detect EMG artifacts in intracranial recordings with the wavelet entropy feature

1.how to detect EMG artifacts in scalp recordings

Example data from 19 channels of scalp EEG are in data.mat.

Calling the Matlab function scalp_example.m will load the data and find the artifacts.

The processed data are saved in the EMGdetClass 'obj'

It produces three variables: chanIdx, startIdxDe, stopIdxDe.

The three Idx files create 1xN vectors for N detected artifacts.

chanIdx:  for each detected artifact, returns the channel number of the event (which corresponds to the row in the data.mat matrix).

startIdxDe: sample number where the artifact starts

stopIdxDe: sample number where the artifact stops

To view the data in channel 'N', do the following:

N=1;  %to view channel 1

data = obj.X(N,:);   % obj.X is the same scalp EEG after common average referencing

plot(data)

hold on

plot(obj.startIdxDe(chanIdx==N),max(data(N,:)),'o')

plot(obj.stopIdxDe(chanIdx==N),max(data(N,:)),'+')

2. how to compute the wavelet entropy feature of HFO detections in intracranial recordings

Example data from 61 channels of intracranial EEG are in intracranialEEG.mat, with corresponding HFO detections in qHFOdetection.mat

Calling the Matlab function example_intracranial_feature.m will compute the 'wavelet entropy' feature (within ‘compute_feature_demo.m’) corresponding to individual HFO detections and then plot a single iEEG channel with labels indicating detections of EMG-false-HFO and real HFOs.

The determination of real vs. EMG-false-HFO uses the threshold of 8.956, as described in the text.